# Basic Workflow

The following pages explore some of PrusaSlicer’s many features, which allow you to customize your prints in a variety of ways. Understanding these tools gives you a high level of control over the printing process and can significantly improve the quality of your results.

That said, the software can feel overwhelming at first, especially for beginners. To make the process more manageable, we will start by guiding you step by step through PrusaSlicer’s basic workflow before building toward a few of the more advanced features.

# Import the Model

Before you can print anything, you need a 3D model. You can either download models from online libraries like Printables or Thingiverse, or create your own using CAD software such as Autodesk Fusion.

First, you need to save your model to a location on your computer that you can easily find.

There are several ways to import the model into PrusaSlicer:

- Click “Add”, which is the only active button in the top menu bar. This will open your file explorer, where you can locate and import your file. (this is my preferred method of adding models)

- Go to File → Import → Import STL/3MF/STEP/OBJ/AMF... this will also open your file explorer.

- Drag and drop a file directly from a folder on your computer into the PrusaSlicer window.

import_model.png

Regardless of whichever method you choose, the model will end up on your print plate.

# Inspect the Model

Before proceeding it is important to inspect your model from all angles to make sure that it imported correctly.

Left-click anywhere outside a model and drag to rotate the view.

rotating_view.webm

Right-click and drag to pan the view.

panning_view.webm

Scroll with the mouse wheel to zoom in and out.

zooming_view.webm

# Position the Model

To ensure a successful print, your model should typically have its largest flat surface touching the print bed.

If the model imported at an angle that is not optimal for printing you can use the rotate tool to manually adjust its orientation, or you can select the Place on Face tool, then click one of the highlighted faces to quickly snap the model into the correct position.

place_on_face1.webm

You can optionally move the model around with your mouse, but leaving it in the middle of the printing surface is usually best. The exact position on the plate does not matter as long as no part of the model is out of the printing area. In that case, the program will warn you, and the model will turn blue.

# Select the Printer, Filament, and Preset Settings

Now, you have to specify three settings from the right panel (preferably in this order):

- the “Printer”

- the “Filament”

- the initial “Print settings”

printer.png

You will primarily have access to Prusa MK4S printers with HF0.4mm nozzles. We do have additional printers and accessories in the makerspace, but those require additional training to be able to use them.

filament_warning.png

The Print settings primarily affect the layer height, which is typically between 0.10 and 0.28 mm. Smaller layer heights offer more detail but take longer to print. A good choice is typically one of the 0.20mm profiles, which provide a good balance of speed and quality for most use cases.

print_settings.png

# Enable Supports

A 3D printer cannot reliably print into thin air. Choose whether to generate supports, i.e. printed “scaffolding” which increases reliability when printing overhangs. Select Supports → Everywhere if you are unsure if the model needs supports or not. This might result in a longer print time and some extra post-processing, but it will ensure that the print doesn’t fail partway through.

supports.png

# Brim and Infill

You also have the option to change the object's infill density (i.e., how much material should be on the inside). However, as we will explain in more detail later, the default infill value of 15% is just fine for most prints.

If only a small part of your object is touching the print bed, you might also want to enable brim - a thin outline around the base of the object that increases the contact area and helps the part stick to the print surface. As with supports, if you are unsure about the amount of print surface contact, enable the Brim setting.

infill_and_brim.png

# Slice and Preview

With these settings selected, you can press the “Slice now” button in the bottom right corner. PrusaSlicer will then generate a G-code preview for you. Depending on the complexity of the model and your hardware specifications, this might take a minute.

This will show you a preview of the sliced model (including supports, brim, etc.) and useful information about print time and material consumption in the bottom right corner. You can move the vertical slider on the right to preview the individual layers of the print. THIS IS VERY IMPORTANT! Doing this can often spot potential problems that you need to fix before attempting to print the model.

slice_and_inspect.webm

# Export and Print

After checking the G-code preview, you can click save the G-code to be transferred to the printer. The preferred method is to first insert a USB drive into the computer. Then you can click the button in the far-right bottom corner of the window to export the G-code directly to the USB. Otherwise, you can use the Export G-code to save the file to your computer, which you can then transfer to the printer later.

export.png

Now, you just have to prepare your printer (i.e., clean the print bed and load the correct filament) and start the print!

At the end of each lesson you will need to conduct a practical demonstration

of your mastery of the material from that lesson.