# Interface: Right Side Panel

Next we will look at the panel on the right side of the screen, which is the most important part of the program - as we saw in the basic workflow page, it is used for every print job. All the fundamental settings are placed in this part of the screen: selecting the layer height, print material, printer type, enabling supports, and choosing the infill density. Below all the settings, there is the Object list, which includes all the models in the project - the rocket engine, in our case.

right_side.png

When setting up a print it is important to change the settings in the following order.

# Printer:

printer.png

Choose the printer for this job. You should always select the printer first, because it overrides all the other settings. For this training we will focus on the standard MK4S. There are other printers and accessories available but those will require additional training before you are able to use them.

# Filament:

filament.png

Different materials require different settings (temperature, print speed, etc.), so it’s important to choose the correct one. We have many different colors of Overture PLA available for you to use in the makerspace. Additionally, we have a few spools of PETG and TPU available if.

If you need to use a different material other than the ones that are provided you need to confirm that it is ok before using it. Some materials like ABS are not allowed to be used in the space.

# Print settings:

print_settings.png

This section allows you to quickly select one of the presets for the given printer and filament. From the dropdown menu, you can choose from various print layer heights. Note that even though these presets are named after the layer height, each of them also includes many other different settings that influence the quality and print time.

The lower the layer height, the better the details, but the print process will take significantly longer. The 0.2 mm layer height usually offers a good tradeoff between quality and speed for most projects.

# Supports:

A 3D printer cannot reliably print into thin air. As the material builds up layer by layer, each layer has to have at least a bit of a solid base underneath.

failed.png

The parts of the model that are hanging or protruding “in the air” are called overhangs. Whether these overhangs are printable depends on their size and angle. Roughly speaking, 45° is okay (imagine the arms of the letter “Y”), but 90° (arms of the letter “T”) is not.

| overhangs.png | letters.png |
| --- | --- |

Generating supports, i.e. printed “scaffolding” increases reliability when printing overhangs. It can be difficult for beginners to assess whether supports are needed. When downloading a model from sites like Printables, the model authors usually leave instructions about whether you should enable them or what other printer settings they recommend, so make sure to read the model descriptions!

Select Supports → Everywhere if you are unsure about the supports and there is no helpful comment on the model page. This might result in a longer print time and some extra post-processing, but it will ensure that the print doesn’t fail partway through.

supports.png

many times you can eliminate the need for supports by altering the orientation of the model to get rid of any overhangs. The letter “T” example from earlier could be done by

# Infill:

| infill_percents.png | 3D models are rarely printed fully solid on the inside, with 100% density. The print would take ages, consume loads of material and the resulting object would be heavy as a brick. Instead, most prints are hollow, with supporting mesh filling the cavity: the infill. Objects with higher infill density are also somewhat tougher. Some mechanical parts are printed with 90-100% infill, however, most of the time it doesn’t make sense to set more than, say, 40% density. 15-20% is usually a good tradeoff between toughness and reliability vs. print time and material consumption. Many times you can get away with as little as 5%. If you want to improve toughness, the better option is usually increasing perimeters - which is considered more effective than adding infill density. We will explore this option in the Object list panel below. |
| --- | --- |

The infill can have various patterns with either aesthetic or mechanical special benefits (e.g. honeycomb, rectangular “checkers”, stars, or a strange curved mesh called the gyroid). The main purpose of infill is to add an internal support structure for the top layers of the print. The two patterns that you will use most frequently are grid and gyroid.

| grid.png | Grid is the standard infill pattern used in most preset settings. It is a reliable choice for many prints because it is quick to produce and uses relatively little filament. Grid works well as a default option, especially for nonfunctional parts where strength is not a primary concern. |
| --- | --- |
| gyroid.png | Gyroid is an advanced infill pattern available in PrusaSlicer. It creates a smooth, continuous 3D structure that provides strength in all directions. Compared to simpler patterns, gyroid offers an excellent balance of strength, flexibility, and material use. It also reduces vibration and produces consistent support throughout the part. While it may take slightly longer to print than grid, gyroid is a great choice for functional parts or prints that need more durability and uniform strength. |

# Brim:

infill_and_brim.png

| brim.png | The Brim option adds a thin layer of material that extends outward from the base of your print, creating a larger surface area that sticks to the build plate. This helps improve bed adhesion and reduces the chances of warping or the print coming loose, especially for parts with small footprints or sharp corners. A brim is typically easy to remove after printing and uses only a small amount of extra filament. A brim is typically needed when printing tall or narrow parts, or when working with materials that are more prone to warping. Like the supports setting, it can be difficult for beginners to recognize when a brim is necessary. If you are unsure, it is usually better to enable a brim than to risk the print failing due to poor bed adhesion. |
| --- | --- |

# Object list panel:

object_list.png

In the Object List panel, you will see all of the models currently placed on the print bed. For simple prints, this list may only contain a single item. For more complex projects, the list can include multiple parts and nested components.

Selecting a model, either directly on the print plate or by clicking its name in the Object List, will open a panel below the list with additional controls. This panel allows you to precisely adjust the position, rotation, and scale of your model by entering exact values, rather than relying on dragging the object manually.

object_manipulation.png

If you make any adjustments to the scale or rotation, you can reverse them by clicking the small circular arrow icon that appears next to the modified values in the Object manipulation panel. Additionally, make sure you keep the ratios locked (using the padlock icon), so all the axes scale uniformly, otherwise the object will deform!

| object_menu.png object_settigns.png | Right-clicking on an object, either on the print plate or in the object list panel, will open a menu with additional settings that allow you to customize and edit your model. We will not go through most of these options in this training, but you are encouraged to explore them on your own and experiment with what they do. The two options we will focus on are “Infill” and “Layers and Perimeters.” Selecting either of these will open a new panel below the object list. The Infill panel gives you greater control over both the infill density and pattern. For most nonfunctional prints, 15%–20% Grid infill is a good default setting. For functional parts that require more strength, you can increase the infill to around 40% using the Gyroid pattern. While you are free to experiment with different patterns, you should generally avoid going above 50% infill, as it provides little additional strength while significantly increasing print time and filament use. The Layers and Perimeters panel allows you to adjust the number of top and bottom layers, the layer height, and the number of perimeters. The layer height has the greatest impact on print quality. You can adjust it between 0.10 mm and 0.30 mm, with 0.20 mm being a reliable setting for most prints. To ensure a successful print, you should use at least 5 top layers, 3 bottom layers, and 2 perimeters. Increasing these values to 6–8 top and bottom layers and 4–6 perimeters can significantly improve the strength and durability of your part. |
| --- | --- |

The final option in the right-side panel is the Slice button at the bottom. This applies all of the settings you have selected for your model and converts it into G-code, which the printer uses to create the part. We will go into more detail about this process in a later section.