# Dimensional Accuracy

# Printer Resolution

3D printers have different resolutions in the vertical and horizontal axis. These resolutions directly affect the dimensional accuracy of the printed part itself.

The horizontal resolution (or XY resolution, parallel to the print surface) is limited by the nozzle diameter. Nozzles on printers vary in size, with the common standard nozzle orifice size being 0.40mm, such as the ones that are installed on our Prusa machines. Smaller or bigger diameter nozzles are also available - such as the 0.25 or 0.6 mm nozzle.

nozzles.jpg

The vertical resolution is limited by the layer height. The smaller it is, the smoother the printed surface will be. But the layer height also directly affects the print time.

layers.jpg

Generally, the vertical resolution will be significantly higher than the horizontal resolution. As a result, the print orientation has a big impact on aesthetic qualities as well as dimensional accuracy.

A good rule of thumb to follow is that the range of acceptable layer heights for a printer is between 25%-75% of the nozzle width. For our printers this would be layer heights between 0.1mm-0.3mm with the most common layer height being 0.2mm. When designing your parts keep these dimensions in mind. No horizonal feature should be smaller than two times diameter of the nozzle (~0.8mm) and no vertical future should be smaller than two times the layer height (~0.4mm).

# Tolerances and Allowances

## This section is currently under construction

Recall that tolerances are the amount of variation between a design and the actual dimensions of a part. 3D printers can be extremely accurate, but a certain threshold of tolerance will always apply. For example you design a cube to be 10mm along each edge and the dimensions of the part that was actually printed are 9.5mm x 10.1mm x 10.0mm.

https://all3dp.com/2/3d-printing-tolerances-test-fdm/

https://maktraequipments.com/blogs/3d-printer/how-much-tolerance-for-3d-printing